EXPERIMENT 1

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.NUMERIC_STD.ALL;

entity ALU1 is

Port (

A      : in  STD_LOGIC_VECTOR (3 downto 0);

B      : in  STD_LOGIC_VECTOR (3 downto 0);

SEL    : in  STD_LOGIC_VECTOR (2 downto 0);

RESULT : out STD_LOGIC_VECTOR (3 downto 0);

CARRY  : out STD_LOGIC

);

end ALU1;

architecture Behavioral of ALU1 is

-- Temporary signal should be 5 bits wide to store carry

signal temp : UNSIGNED(4 downto 0);

begin

process(A, B, SEL)

begin

case SEL is

when "000" =>  -- ADD

temp   <= ('0' & unsigned(A)) + ('0' & unsigned(B));

RESULT <= std_logic_vector(temp(3 downto 0));

CARRY  <= temp(4);

when "001" =>  -- SUB

temp   <= ('0' & unsigned(A)) - ('0' & unsigned(B));

RESULT <= std_logic_vector(temp(3 downto 0));

CARRY  <= temp(4);

when "010" =>  -- AND

RESULT <= A and B;

CARRY  <= '0';

when "011" =>  -- NAND

RESULT <= not (A and B);

CARRY  <= '0';

when "100" =>  -- OR

RESULT <= A or B;

CARRY  <= '0';

when "101" =>  -- XOR

RESULT <= A xor B;

CARRY  <= '0';

when "110" =>  -- XNOR

RESULT <= A xnor B;

CARRY  <= '0';

when "111" =>  -- PASS A

RESULT <= A;

CARRY  <= '0';

when others =>

RESULT <= (others => '0');

CARRY  <= '0';

end case;

end process;

end Behavioral;

TEST BENCH

LIBRARY ieee;

USE ieee.std_logic_1164.ALL;

-- Uncomment the following library declaration if using

-- arithmetic functions with Signed or Unsigned values

USE ieee.numeric_std.ALL;

ENTITY EXTB IS

END EXTB;

ARCHITECTURE behavior OF EXTB IS

-- Component Declaration for the Unit Under Test (UUT)

COMPONENT ALU1

PORT(

A : IN  std_logic_vector(3 downto 0);

B : IN  std_logic_vector(3 downto 0);

SEL : IN  std_logic_vector(2 downto 0);

RESULT : OUT  std_logic_vector(3 downto 0);

CARRY : OUT  std_logic

);

END COMPONENT;

--Inputs

signal A : std_logic_vector(3 downto 0) := (others => '0');

signal B : std_logic_vector(3 downto 0) := (others => '0');

signal SEL : std_logic_vector(2 downto 0) := (others => '0');

--Outputs

signal RESULT : std_logic_vector(3 downto 0);

signal CARRY : std_logic;

-- No clocks detected in port list. Replace <clock> below with

-- appropriate port name

BEGIN

-- Instantiate the Unit Under Test (UUT)

uut: ALU1 PORT MAP (

A => A,

B => B,

SEL => SEL,

RESULT => RESULT,

CARRY => CARRY

);

-- Stimulus process

stim_proc: process

begin

-- hold reset state for 100 ns.

A <= "0101";  -- 5

B <= "0011";  -- 3

-- Perform all 8 operations

SEL <= "000"; wait for 20 ns;  -- ADD

SEL <= "001"; wait for 20 ns;  -- SUB

SEL <= "010"; wait for 20 ns;  -- AND

SEL <= "011"; wait for 20 ns;  -- NAND

SEL <= "100"; wait for 20 ns;  -- OR

SEL <= "101"; wait for 20 ns;  -- XOR

SEL <= "110"; wait for 20 ns;  -- XNOR

SEL <= "111"; wait for 20 ns;  -- PASS A

-- insert stimulus here

wait;

end process;

END;