EXPERIMENT 2

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.NUMERIC_STD.ALL;  -- for arithmetic and conversions

entity fifo_exp is

Port (

clk      : in  STD_LOGIC;

rst      : in  STD_LOGIC;

en       : in  STD_LOGIC;

w        : in  STD_LOGIC; -- write enable

datain   : in  STD_LOGIC_VECTOR(7 downto 0);

dataout  : out STD_LOGIC_VECTOR(7 downto 0);

read     : out STD_LOGIC; -- empty flag

full     : out STD_LOGIC  -- full flag

);

end fifo_exp;

architecture Behavioral of fifo_exp is

type MEM_ARRAY is array (0 to 15) of STD_LOGIC_VECTOR(7 downto 0);  -- 16x8 FIFO

signal MEM    : MEM_ARRAY := (others => (others => '0'));

signal WPTR   : unsigned(3 downto 0) := (others => '0');  -- Write pointer

signal RPTR   : unsigned(3 downto 0) := (others => '0');  -- Read pointer

signal empty  : STD_LOGIC := '1';

signal full_i : STD_LOGIC := '0';

begin

process(clk, rst)

begin

if rst = '1' then

WPTR    <= (others => '0');

RPTR    <= (others => '0');

MEM     <= (others => (others => '0'));

dataout <= (others => '0');

empty   <= '1';

full_i  <= '0';

elsif rising_edge(clk) then

if en = '1' then

-- WRITE operation

if w = '1' then

if full_i = '0' then

MEM(to_integer(WPTR)) <= datain;

WPTR <= WPTR + 1;

empty <= '0';

end if;

if WPTR + 1 = RPTR then

full_i <= '1';

else

full_i <= '0';

end if;

-- READ operation

else

if empty = '0' then

dataout <= MEM(to_integer(RPTR));

RPTR <= RPTR + 1;

full_i <= '0';

if RPTR + 1 = WPTR then

empty <= '1';

end if;

else

dataout <= (others => '0');

end if;

end if;

end if;

end if;

end process;

read <= empty;

full <= full_i;

end Behavioral;

TEST BENCH

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.NUMERIC_STD.ALL;

entity fifo_tb is

end fifo_tb;

architecture behavior of fifo_tb is

-- Component Declaration for the Unit Under Test (UUT)

component fifo_exp

port (

clk      : in  std_logic;

rst      : in  std_logic;

en       : in  std_logic;

w        : in  std_logic;

datain   : in  std_logic_vector(7 downto 0);

dataout  : out std_logic_vector(7 downto 0);

read     : out std_logic;

full     : out std_logic

);

end component;

-- Signals

signal clk      : std_logic := '0';

signal rst      : std_logic := '0';

signal en       : std_logic := '0';

signal w        : std_logic := '0';

signal datain   : std_logic_vector(7 downto 0) := (others => '0');

signal dataout  : std_logic_vector(7 downto 0);

signal read     : std_logic;

signal full     : std_logic;

-- Clock period definition

constant clk_period : time := 10 ns;

begin

-- Instantiate the Unit Under Test (UUT)

uut: fifo_exp

port map (

clk      => clk,

rst      => rst,

en       => en,

w        => w,

datain   => datain,

dataout  => dataout,

read     => read,

full     => full

);

-- Clock process

clk_process : process

begin

clk <= '0';

wait for clk_period / 2;

clk <= '1';

wait for clk_period / 2;

end process;

-- Stimulus process

stim_proc: process

begin

-- Initial Reset

rst <= '1';

en  <= '0';

w   <= '0';

wait for 20 ns;

rst <= '0';

en  <= '1';

-- WRITE Phase

report "----- START WRITING TO FIFO -----";

w <= '1';

for i in 0 to 7 loop

datain <= std_logic_vector(to_unsigned(i + 10, 8));  -- Write data 10..17

wait for clk_period;

end loop;

w <= '0';

wait for 30 ns;

-- READ Phase

report "----- START READING FROM FIFO -----";

for j in 0 to 7 loop

wait for clk_period;

end loop;

-- Wait for observation

wait for 50 ns;

report "----- TEST COMPLETE -----";

wait;

end process;

end behavior;