DHT11

#include "DHT.h"

#define DHTPIN 7          // DHT11 data pin connected to digital pin 7

#define DHTTYPE DHT11     // Define the sensor type (DHT11)

DHT dht(DHTPIN, DHTTYPE); // Initialize DHT sensor

void setup() {

Serial.begin(9600);

Serial.println("DHT11 Temperature and Humidity Sensor");

dht.begin();

}

void loop() {

delay(2000);  // Wait 2 seconds between readings

float humidity = dht.readHumidity();     // Read humidity

float temperature = dht.readTemperature(); // Read temperature in °C

// Check if readings failed

if (isnan(humidity) || isnan(temperature)) {

Serial.println("Failed to read from DHT sensor!");

return;

}

Serial.print("Humidity: ");

Serial.print(humidity);

Serial.print(" %\t");

Serial.print("Temperature: ");

Serial.print(temperature);

Serial.println(" °C");

}

Ultrasonic

#define TRIG_PIN 7  // Trigger pin

#define ECHO_PIN 6  // Echo pin

void setup() {

Serial.begin(9600);

Serial.println("Ultrasonic Sensor Test");

pinMode(TRIG_PIN, OUTPUT);

pinMode(ECHO_PIN, INPUT);

}

void loop() {

long duration;

float distance;

// Clear the trigger pin

digitalWrite(TRIG_PIN, LOW);

delayMicroseconds(2);

// Send a 10µs HIGH pulse to trigger the sensor

digitalWrite(TRIG_PIN, HIGH);

delayMicroseconds(10);

digitalWrite(TRIG_PIN, LOW);

// Measure the time it takes for the echo to return

duration = pulseIn(ECHO_PIN, HIGH);

// Calculate distance in centimeters

distance = duration * 0.034 / 2;

Serial.print("Distance: ");

Serial.print(distance);

Serial.println(" cm");

delay(500); // Wait half a second before next measurement

}

Relay and buzzer

#define RELAY_PIN 5   // Relay connected to pin 5

#define BUZZER_PIN 4  // Buzzer connected to pin 4

void setup() {

pinMode(RELAY_PIN, OUTPUT);

pinMode(BUZZER_PIN, OUTPUT);

Serial.begin(9600);

Serial.println("Relay and Buzzer Test");

}

void loop() {

// Turn ON relay and buzzer

digitalWrite(RELAY_PIN, HIGH);  // Energize relay

digitalWrite(BUZZER_PIN, HIGH); // Turn on buzzer

Serial.println("Relay and Buzzer ON");

delay(2000); // ON for 2 seconds

// Turn OFF relay and buzzer

digitalWrite(RELAY_PIN, LOW);

digitalWrite(BUZZER_PIN, LOW);

Serial.println("Relay and Buzzer OFF");

delay(10000); // OFF for 2 seconds

}

Dht11 and relay

#include "DHT.h"

#define DHTPIN 7        // DHT11 data pin

#define DHTTYPE DHT11   // Define sensor type

#define RELAY_PIN 6     // Relay control pin

DHT dht(DHTPIN, DHTTYPE);

void setup() {

Serial.begin(9600);

Serial.println("DHT11 with Relay Control");

dht.begin();

pinMode(RELAY_PIN, OUTPUT);

digitalWrite(RELAY_PIN, LOW);  // Make sure relay starts OFF

}

void loop() {

delay(2000); // Wait 2 seconds between readings

float humidity = dht.readHumidity();

float temperature = dht.readTemperature(); // Celsius

if (isnan(humidity) || isnan(temperature)) {

Serial.println("Failed to read from DHT sensor!");

return;

}

Serial.print("Temp: ");

Serial.print(temperature);

Serial.print(" °C  |  Humidity: ");

Serial.print(humidity);

Serial.println(" %");

// --- Relay control logic ---

if (temperature > 30.0) {  // Threshold: 30°C

digitalWrite(RELAY_PIN, HIGH);  // Turn relay ON

Serial.println("Relay ON (Temperature High)");

} else {

digitalWrite(RELAY_PIN, LOW);   // Turn relay OFF

Serial.println("Relay OFF (Temperature Normal)");

}

}

Led and switch

#define LED_PIN 6       // LED connected to pin 6

#define SWITCH_PIN A1   // Push button connected to pin 0

void setup() {

pinMode(LED_PIN, OUTPUT);       // Set LED pin as output

pinMode(SWITCH_PIN, INPUT_PULLUP); // Enable internal pull-up for the switch

Serial.begin(9600);

Serial.println("LED and Switch Test");

}

void loop() {

int switchState = digitalRead(SWITCH_PIN); // Read switch state

// The pin will read LOW when the button is pressed (due to INPUT_PULLUP)

if (switchState == LOW) {

digitalWrite(LED_PIN, HIGH); // Turn LED ON

Serial.println("Button Pressed - LED ON");

} else {

digitalWrite(LED_PIN, LOW);  // Turn LED OFF

Serial.println("Button Released - LED OFF");

}

delay(100); // Small delay to stabilize reading

}

IR sensor

#define IR_SENSOR_PIN 7    // IR sensor output pin

#define LED_PIN 6      // LED pin (built-in LED on Arduino)

void setup() {

pinMode(IR_SENSOR_PIN, INPUT);  // Set sensor pin as input

pinMode(LED_PIN, OUTPUT);       // Set LED pin as output

Serial.begin(9600);

Serial.println("IR Object Detection Test");

}

void loop() {

int sensorState = digitalRead(IR_SENSOR_PIN); // Read the sensor output

if (sensorState == LOW) { // LOW means object detected (for most IR modules)

digitalWrite(LED_PIN, HIGH);  // Turn on LED

Serial.println("Object Detected!");

} else {

digitalWrite(LED_PIN, LOW);   // Turn off LED

Serial.println("No Object");

}

delay(300); // Small delay to stabilize readings

}

Led

// Simple LED blink for pins 6 and 8

int led1 = 6;   // LED connected to pin 6

int led2 = 7;   // LED connected to pin 8

void setup() {

pinMode(led1, OUTPUT);  // Set pin 6 as an output

pinMode(led2, OUTPUT);  // Set pin 8 as an output

}

void loop() {

digitalWrite(led1, HIGH);  // Turn on LED on pin 6

digitalWrite(led2, LOW);   // Turn off LED on pin 8

delay(500);                // Wait 0.5 seconds

digitalWrite(led1, LOW);   // Turn off LED on pin 6

digitalWrite(led2, HIGH);  // Turn on LED on pin 8

delay(500);                // Wait 0.5 seconds

}

And or code

void setup() {

// Configure pin modes

pinMode(6, OUTPUT);   // LED output

pinMode(A4, INPUT);   // Input 1

pinMode(A5, INPUT);   // Input 2

}

void loop() {

// Read digital inputs

int val  = digitalRead(A4);

int val1 = digitalRead(A5);

// If both inputs are HIGH

if (val == 1 && val1 == 1) {

digitalWrite(6, HIGH);   // Turn ON LED

} else {

digitalWrite(6, LOW);    // Turn OFF LED

}

}

Or

void setup() {

// Configure pin modes

pinMode(6, OUTPUT);   // LED output

pinMode(A0, INPUT);   // Input 1

pinMode(A1, INPUT);   // Input 2

}

void loop() {

// Read digital inputs

int val  = digitalRead(A0);

int val1 = digitalRead(A1);

// OR gate condition

if ((val == 1) || (val1 == 1)) {

digitalWrite(6, HIGH);   // LED ON if either input is HIGH

} else {

digitalWrite(6, LOW);    // LED OFF if both inputs are LOW

}

}